# ReadMe

This pre-loaded starter library is designed so that you can start work straight away. It contains a number of parts ready for schematic capture and placement in PCB. However, it does not contain any simulation data for either digital or analog simulation. Should you require the ability to simulate parts you have the option of the HyperLynx Analog libraries shipped with the HyperLynx Analog product, or you can download the much larger Starter Library from Optimum Design Associates (ODA) directly from the Start Page of xDX Designer:

This library is fully compatible with the ODA library so you may copy parts between the two or replace entire partitions in either (Integrated flow with Central Library). It is structured as a series of categorized folders (net list flow) or symbol partitions (Integrated central library flow).

The library contains a number of symbols with both ANSI and IEC representations, for example the Resistor symbol - Resistor.1 and Resistor.2 as ANSI and IEC respectively. Other symbols that include both representations include the logic symbols in the IC library.

The library is intended to be used in conjunction with xDX Databook and contains an MS Access database with full parametric data for all of the parts in the library, it is particularly important that you place the resistors and capacitors into your schematics via xDX Databook to ensure they have the necessary packaging information attached.

# Support for SQLite

In the PADS VX.2 and later releases we have added support for SQLite databases. SQLite is a free database and there are a number of tools available on the web to allow you to create and manage SQLite databases (see www.sqlite.org). One advantage with SQLite is that it does not rely on users configuring an ODBC connection to the database, providing simplified access for users.

SQLite is fully integrated with the central library in the PADS Standard Plus integrated project flow and you may connect to a SQLite database and add parametric data in the same way as when connecting to a Microsoft Access or other database source, using the Edit Parametric Data from the Navigation tree in the library tools:

For users of the netlist flow there are a number of editing tools and addons for common web browsers such as Firefox (https://addons.mozilla.org) and Chrome and standalone tools (such as SQLite Pro http://sqlite2009-pro.software.informer.com/3.7/) that enable you to edit the content of SQLite databases.

To enable you to take advantage of SQLite with the shipped starter libraries we provide an SQLite version of the Access database and a pair of preconfigured Databook control files (*.dbc) in the ConvertDBC.exe file in the library tree.

These are direct replacements for the existing MS Access compatible ‘dbc’ files supplied with the libraries. Exit from xDX Designer and run the ConvertDBC.exe file (double click the file), accept the option to overwrite the existing files.

Next time you invoke PADS, PADS Databook will use the SQLite database in place of the Access version.